DATA LINK LAYER & MEDIUM ACCESS SUB-LAYER

Error Detection & Correction

Data link layer:-

- Each frame must be distinguishable from each other.

- Data is between header & trailer.

- Header: A code which helps in transmitting data to next node.

- Trailer: A code used for error detection & correction.

Protocols at data link layer:-

- Byte oriented protocol

- Bit oriented protocol

Sample frame of data link layer:-

- Flag: A fixed pattern used for indicating starting & ending of a flag.

- Header contains MAC address of source & destination + control flow info.

Byte Oriented Protocol

- This protocol reads data in terms of bytes.

- In this, data contains certain control patterns & thus extra bytes are stuffed into it in order to avoid conflict.

- We call these stuffed data as ESC.

- ESC: Extra stuffed characters.

- If ESC is found in data, then additional ESCs are stuffed. So that when ESC-Flag or ESC-ESC pattern is noticed, receiving device can understand that it is part of data.

Bit Oriented Protocol

- When code 0111 1110 appears, it is mistaken as a flag.

- So after each 5th 1, a 0 is stuffed which is removed at receiver’s end.

Types of Errors

- Single bit error: *As the name says.*

- Burst error: Errors in which more than two bits are changed.

Error Correction

- For correcting the error, some extra bits of data are sent.

Detection v/s Correction

| Detection | Correction |
| --- | --- |
| Not much difficult. | Is difficult. |
| Just detects if error exists or not, magnitude is not a concern. | Gotta know the size & location of the error with some mathematical calculations. |

Error Correction Methods

- Forward error correction

- Retransmission

- XORing

Forward Error Correction Method

- Receiver guesses the correct data seeing the redundant bits.

- Redundant bits: Bits inserted into original data in order to avoid errors.

- This method is used generally in wireless transmission cases due to high burst error length.

- For example:

Data = 0011 = 0 0 1 1

After adding redundant bits, data = 000 000 111 111

- So, more 0s in a pack of 3 (pack size may vary) denotes 0.

- And similarly, more 1s in a pack denotes value as 1.

Let’s say that 000 000 111 111 becomes 001 000 111 111

Third letter of first pack is changed as an error.

But because majority values in pack are 0, it is considered 0.

Hence, 001 000 111 111 = 0 0 1 1 = 0011

Retransmission Method

- This method is used when error length is less.

- Receiver detects if an error is occurring or not.

- If yes, then requests the transmitter for resending the data.

- When the data is found error free, the process is terminated.

Redundancy Codes

Types of redundancy coding schemes:-

- Block coding

- Convolution coding

XORing

- This is done by bitwise XOR operation between 2 consecutive transmissions of same data.

- For example:

Data 1 = 0010, Data 2 = 0010

Now, (Data 1) XOR (Data 2) = 0000	[All 0 bits means no error]

Data 1 = 1011, Data 2 = 1001

Now, (Data 1) XOR (Data 2) = 0010	[Even one 1 found means error]

Block Coding

- The code is divided into group of k-bits.

- Data words: *k above*

- And then redundancy bits are added to each group, so n = k + r

- Code word: *n above*

- Can’t detect error of more than 2 bits.

- For example:

Let’s say that k=2, n=3 as shown above in the table.

For 01 -> 011, 01 is detected and thus is passed successfully.

But if it is 000 instead, the error is of more than 2-bits making it undetectable.

It is undetected because 000 exists in the table for another value.

- Another example:

In this, user receives a corrupted codeword instead 01011.

Then user manually matches it with each codeword from table, finding the difference being of 2-bits.

Whereas the match with second codeword is of 1-bit difference.

And comes to know that in which codeword the error was.

Linear Block Codes

- Subset of block codes.

- In this, when we calculate XOR of two valid codewords, we get another valid codeword.

Parity Check Code

- A type of simple block code.

- Hence, number of digits in codeword is one more than dataword.

n = k + 1

- Value of syndrome is 0 if 1s in codeword are even (including parity bit).

- And for odd 1s it is 1.

- The parity bit is chosen the same way.

Cyclic Redundancy Check (CRC)

- Method used to check if any accidental change is made to signal during transmission.

- This error occurs in communication channel.

- There is a generator polynomial on sender’s and receiver’s side, telling how data was transmitted.

- Example:

Message received: 1010000

Given generator polynomial: x3 + 1

x3 + 1 = x3 + x0 = 1.x3 + 1.x0 = 1.x3 + 0.x2 + 0.x1 + 1.x0 = CRC (1001)

CRC is of 4-bits.

So, append 4 - 1 = 3 zeroes in end of our received message.

@ means XOR:

Checksum

- Data is divided into equal number of bits.

- When before being sent, are segments are complemented by 1 and added, and then their sum is complemented.

- This gives us checksum, which is sent along the data segments.

- After the receiver receives them, same procedure is applied to decode them.

- If it is zero, only then accepted.

Haming Code

Sender’s end:-

- No limit on the length of data to be checked.

- Used for detecting & correcting single bit errors only.

- Parity bits: All bit positions with serial number equal to power of 2.

- Data bits: Bit positions not equal to power of 2.

- For P1: 1-bit is checked, 1-bit is skipped, *repeat*	(1,3,5,7)

- For P2: 2-bit is checked, 2-bit is skipped, *repeat*	(2,3,6,7)

- For P4: 4-bit is checked, 4-bit is skipped, *repeat*	(4,5,6,7)

Receiver’s end:-

- If value of all parity bits at receiver’s end is zero, then its free of error.

- Else it is not.

Correction:-

- Step 1: Parity bits are written together in series.

- Step 2: This combination is converted into decimal value.

- Step 3: Then the bit at serial number equal to decimal number obtained, is inverted.

Flow & Error Control

- Data link layer ensures flow & error control.

- Flow control: Controlling amount of unacknowledged data to be sent by sender.

- Error at data link layer is controlled by retransmission of data.

- Protocols at this level are implemented using programming languages.

- Noiseless channel: Channel through which, no data are corrupted while travelling.

- Noisy channel: *Now you know*

- ARQ: Automatic repeat request

Noiseless: Simplex Protocol

- Different data frames are sent parallelly, bit by bit, as separate events.

- And definitely, these frames take time to reach to the receiver.

Noisy: Stop and Wait ARQ

- Error correction is done by keeping a copy of the frame, which is sent when the acknowledgement time expires.

- These frames being sent are given serial numbers.

- Acknowledgement is given frame by frame & next frame is sent only after the acknowledgment of previous frame.

- Frame is resent even when the acknowledgment fails to reach the sender.

- After getting the acknowledgement, the copy of frame is deleted.

- Bandwidth of channel in “Stop and Wait ARQ” is around 1 Mbps.

- 1 bit of data takes around 20 milliseconds to be sent.

Noisy: Sliding Window Protocols (ARQ)

- Same as stop and wait but sender can send multiple frames at a time.

- Also known as windowing.

- This protocol is also used in TCP.

- These messages however passed through buffers.

- Sender has a buffer called sending window & receiver has a buffer called receiving window.

- Modulo-N: All preceding whole numbers to a given number N.

- Window size: The number of frames a sending or receiving window can store.

- These frames stored in buffers are given sequence numbers as per modulo-N in binaries, N is the number of frames.

- Or simply saying, they are given sequence numbers just like indexes in programming.

For example:

Types:-

Sliding Window Protocol: Go Back N (ARQ)

- In Go Back N protocol; if even a single frame is corrupted or lost, then all frames are resent.

- The sending frame in it is N, whereas receiving frame is 1.

- It also works on the time expiration principle.

Sliding Window Protocol: Selective Repeat (ARQ)

- In this, the size of sender and receiver window are same.

- Sliding window size: Number of frames that could be sent at a time.

- If a corrupt frame is received, then frames are not discarded immediately.

- Rather a negative acknowledgement is sent when a frame mismatches.

- And then that particular frame is resent immediately, without waiting for the expiration time.

Piggybacking

- A technique in which data can flow bidirectionally.

- Unlike previous techniques with a dedicated sender & receiver.

- Acknowledgement (ACK) & negative acknowledgement (NAK) are transmitted along.

- Separate frames containing ACK or NAK are sent.

- These frames are of few bits & called acknowledgement field.

- Station: Points which can receive and send data.

Working principles:-

- If a station has to send both data and acknowledgement, then the acknowledgement is sent along the data.

- This acknowledgement contains the serial number of the frame with it.

- If there is only an acknowledgement to be sent, then it waits for some time to see if any data frame is about to be sent.

- If yes, then it piggybacks (packs/stuffs) the acknowledgement with it.

- Else it sends a separate acknowledgement frame.

- If its only data frame to be sent, then this data frame is piggybacked with last acknowledgement; which is discarded by the receiving station.

- Or alternatively, it is piggybacked with an acknowledgement saying no acknowledgement.

Multiple Access Protocols (MAP)

- Data link layer is responsible for ensuring data transmission between two nodes.

Main functions of MAP are:-

- Data link control

- Multiple access control

Data Link Control

- Ensures reliable transmission of data.

- Uses techniques like framing, error control & flow control.

Multiple Access Control

- Link: Channel

- If there is a dedicated link between two nodes, then data link layer is enough.

- Otherwise a link is concurrently used by multiple nodes.

- Multiple access control is required to avoid signal collision in a common link.

- And it also avoids mismatched data delivery.

Types of network links:-

- Point-to-point link: A sender at one end & a receiver at another end.

- Broadcast link: Multiple senders and receivers are linked to common broadcast channel.

- Word broadcast because the frame transmitted by any sender is received by all receivers, as a copy.

Random Access Protocols (RAP)

- In this protocol, there is no priority based arrangement of nodes for sending data.

- All nodes are given equal priority.

- However, data transmission depends on whether a medium is busy or idle.

- There is no fixed time for sending data.

- There is also no fixed sequence of stations for sending data.

Examples of RAP:-

- Pure ALOHA

- Slotted ALOHA

- CSMA, CSMA/CD, CSMA/CA

Pure ALOHA Protocol

- Throughput on pure ALOHA is maximized when frame length are similar.

Throughput = S = G*(e-2G)

- Throughput is maximum at G = ½

- Pure ALOHA allows senders to send data whenever they have them.

- If a collision happens, the sender resends data after sometime (random time).

- Contention system: Systems with common channel & thus vulnerable to conflicts.

- Concurrent attempt to access the medium can lead to distorted signals.

- Overlapping bits of two separate frames have capability to destroy each other.

Slotted Aloha Protocol

- Created to overcome pure Aloha’s shortcomings.

- Time is divided into slots.

- Different slots are given to different stations.

- When their number comes, they send a frame during their respective slots.

- If any station misses the transmission, then it has to wait for next round of its slot.

- As we can see, complete overlap of bits won’t cause collision.

Throughput = S = G*(e-G)

- Throughput is maximum at G = 1.

CSMA/CD

- CSMA/CS: CSMA with collision detection.

- When a shared channel is idle, two stations may begin transmitting data simultaneously.

- When this happens, these two stations will detect collision immediately.

- And when collision is detected by CSMA/CD protocol, a jamming signal is sent to abort transmission.

- Aborting transmission at such situation saves both time and bandwidth.

- Bandwidth: Transmission energy.

CSMA/CA

- In this variant of CSMA, a station receives signal sent to it despite collision detection.

- When there is no collision, a station receives the signal sent to it.

- But when there is a collision, the station receives signals from both the senders.

- In wired networks, the detection is easy because a lot of energy is emitted in it.

- However, the detection becomes difficult in wireless network.

- Thus, CSMA/CA in wireless network avoids collision, rather than detecting it.