NETWORK LAYER

Switching Techniques

- In large networks, the sender to receiver path may not be direct.

- Switching technique: A technique for deciding best route for data transmission.

- It enables one-to-one communication among systems.

Circuit Switching

- This technique is used for creating a dedicated path between sender & receiver.

- And this path exists until the connection between them is terminated.

- Telephones work on the same principle.

Advantages:-

- Reliable

- Fixed bandwidth

Disadvantages:-

- It takes a long time to be established.

- Expensive

Message Switching

- In this switching technique, whole data is transmitted at once.

- But they are passed from connected nodes to reach the destination.

- Data are stored & forwarded from those nodes.

- Also known as store and forward network.

Advantages:-

- Efficient

- Low bandwidth requirement.

- Less chances of traffic congestion; as data are not continuously transmitted, but stored in intermediate nodes.

- Priority based network.

Disadvantages:-

- Insufficient storage for storing multiple data.

- Transfer delay due to many storing sessions.

Packet Switching

- In this method, data are split into small pieces & then transmitted.

- Different pieces may take separate route to destination.

- After reaching the destination, they are arranged in the required order; using a unique identity number that they were given.

- If any piece of data is missing or corrupted, the sender resends the data.

- Else it receives an acknowledgement.

Advantages:-

- Cost effective

- Reliable

- No traffic congestion

Disadvantages:-

- High delay

- Low quality

- Expensive

- High overhead

Logical Addressing

- IPv4 Address: 32-bit IP address.

- Two devices connected to internet can’t have same address at the same time.

Ex:	Binary:		01110101 10010101 00011101 00000010

Decimal:	117.149.29.2

Classful Addressing

IP address sub-classes:-

- *Notice that its byte above, not bit*

- Network ID: Represents number of networks.

- Host ID: Represents number of hosts.

Classes

Class A:-

- Used in networks with large number of hosts

- First bit is always 0.

Class B:-

- Used networks with small to large number of hosts.

- First bit is always 0.

- Second bit is always 1.

Class C:-

- Used in networks with small number of hosts.

- First bit is always 1.

- Second bit is always 1.

- Third bit is always 0.

Class D:-

- Reserved for multicasting addresses.

- Multicasting addresses: A logical address for a group of hosts.

- It doesn’t support subnetting.

- Subnet: Part of the network path.

- First bit is always 1.

- Second bit is always 1.

- Third bit is always 1.

- Fourth bit is always 0.

Class E:-

- Used for research purposes by developers, consumers can’t access it.

- It doesn’t support subnetting.

- First bit is always 1.

- Second bit is always 1.

- Third bit is always 1.

- Fourth bit is always 1.

Why Use IPv6?

- It supports subnetting.

- It supports supernetting.

- Supernetting: Collection of multiple class Cs to create even larger network.

- Works with less internet resources, tackling resource depletion issues.

Classless Addressing

- Addresses are kept in blocks but without any classes.

- When an entity has to connect to the internet, a block is given to it as per its size.

- Despite solutions like classful & classless addressing, DHCP address depletion still remains a big issue.

- DHCP: Dynamic host configuration protocol, which automatically assigns IP address to users connected to the internet. Without manual configuration.

IPv6

- It is 128-bits long.

- Hexadecimal colon notation: 128-bit is divided into eight parts in this notation.

- Means each part represents 16-bits i.e. 2 bytes.

- And 2-byte data can be represented with four hexadecimal digits.

- Thus, there are total of 32 hexadecimal digits.

- Each 4-digited hexadecimal number is separated by colon (:).

- Address space: Number of addresses possible (like IPv6 can take 2128 addresses).

- Unicast address: Unique address used to send data to a particular computer.

- *MAC address just identifies a computer uniquely*

- Multicast address: An address used to deliver data to each host in a group connected to common network.

- *Note: Host & clients are different*

- Reserved addresses: These addresses are used by various protocols for ensuring proper working of internet.

- They start with eight zeroes (00000000).

IPv4 Header

- IP receives data from 4th layer of OSI model.

- Then divides the data into various packets.

- Then it encapsulates those data & adds its own header information on it.

- This header contains various routing & addressing information.

Short comings:-

- Doesn’t solve the address depletion issue.

- High transmission delay, making it unsuitable for real-time audio & video streaming.

- Doesn’t provide security facilities like encryption & authentication.

IPv6 Header

- Each packet consists of base header & payload.

- Base header contains the same header information & occupies 40 bytes.

- Payload contains extension for header & data.

- And it can occupy upto 65,535 bytes of information.

Transition From IPv4 to IPv6

Three transition strategies:-

- Dual stack

- Tunneling

- NAT protocol translation

Dual stack:-

- Router uses both IPv4 & IPv6 configuration.

- And both point & work to their respective network clouds/spaces.

Tunneling:-

- A better solution.

- User’s data can pass through non-supported IP address & reach its destination.

NAT protocol translation:-

- NAT-PT means network address translation – protocol translation.

- In this, the NAT enabled device routes the data to the network cloud as per the IP version it used.

Address Resolution Protocol (ARP)

- When communicating, a host requires MAC address of the destination device.

- And this device must be connected to the same network channel.

- MAC address is also known as layer-2 address.

- If NIC is changed due to some fault, it results in change of MAC address.

- So, in order to communicate on internet again; the IP address needs to be refreshed.

Broadcasting conversation using ARP:-

- Address routing protocol (ARP): Allows packet to contain host’s IP address.

- To initiate a broadcast message, the device checks who has its IP address in the same network, requesting for conversation using ARP.

- Then the devices having it reply back with ARP.

- Then sender broadcasts the message to their MAC addresses using ARP.

- ARP is transmitted along with data package when communicating.

- Then for the rest of the conversation, devices use their own IP address to send & receiver’s MAC address to receive the data.

- These MAC to IP mappings of both sender & receiver is saved as cache.

- And this is only when they communicate for the first time.

- Rest of the time, they don’t require each other’s IP or MAC addresses.

- These conversations can be done using ARP cache, produced after first conversation.

Dynamic Host Configuration Protocol (DHCP)

*Also discussed earlier*

- Can be used in both local & enterprise servers, irrespective of network size.

- DHCP is a default protocol used by most networking devices; including routers.

- DHCP is also known as RFC (request for comment).

Bootstrap Protocol (BOOTP)

- Used by a client for obtaining IP address of a server.

- Earlier it was made to save a computer’s network connections before booting up.

- One BOOTP server can serve as host on multiple subnets.

Delivery

- Direct delivery: The network shared by the sending & receiving hosts is same.

- Indirect delivery: The packet goes router to router until it reaches the destination host.

Forwarding

- Forwarding: Placing a packet on the route leading to its destination.

- This is done by checking various destinations on a table.

- But this method is impossible these days due to large number of entities, making it difficult to make each entry on the table.

Routing

- Routing: Selection of path for transferring packets from sender to destination.

- Routing is done by router.

- Router decides path as per the information on packet’s header & forwarding table.

Types of routing:-

- Static routing

- Default routing

- Dynamic routing

Routing tables:-

- Static routing table: Contains manual information entries.

- Dynamic routing table: Automatically updated when a change occurs in the internet.

Intra and inter domain routing:-

- Autonomous system (AS): Group of networks & routers under single authority.

- Intra-domain routing: Routing inside the autonomous system.

- Inter-domain routing: Routing between many autonomous systems.

Types of Routing Protocols

Distance Vector Routing (DVR)

- In this type of routing, the cheapest route between two nodes; is the shortest route too.

- Each node maintains a routing table telling shortest route to other nodes from it.

- This table has a column called cost, which is less is nearby nodes.

- So, data travels through the shortest route from node to node, saving time.

- This continuous short path traversal is known as next-hop routing.

Issues with DVR:-

- Sometimes when travelling from node to node, the packet is lost.

- Then the distance between the current node & next node becomes infinity on table.

*Colour white means current node, grey is distance & yellow is next node*

Link State Routing

- In this, the nodes contain all information about the topology/structure in their table.

- This information includes nearby connections, cost, type of connection etc.

- And thus, it uses Dijkstra’s algorithm to build a separate routing table for the nodes.

- However, the calculations might be different for each node; because each node calculate on the basis of their individual perspective on links to other nodes.

Path Vector Routing

- Distance vector routing & link state routing are only used for intradomain communication.

- This is because interdomain requires high scalability, which is absent in both of them.

- Both of these routing methods can’t be tracked when travelling large distances.

- Distance vector routing becomes unstable with multiple hops.

- Link state routing requires huge amount of information for calculating routing table.

- Thus, they create high overhead.

- And that’s when comes another routing protocol called path vector routing.

- BGP: A path vector protocol used for inter-domain communication.

- The nodes receive the distance vector instead of the router.

- This is done so that node can do routing calculations.