TRANSPORT LAYER

Transport Layer Services & Protocol

- It is responsible for reliable transportation of data.

- Transport layer is more abstracted (high-level) than network layer.

- Sender’s side – Breaks application messages into small pieces & passes them to network layer.

- Receiver’s side – Joins the pieces into message again & passes to session layer.

- For example: TCP, UDP etc.

Process to Process Communication

- Data link layer is responsible for transmitting frames between two nodes.

- Network layer is responsible for delivering datagrams between two hosts.

- Datagram: Head, tail & main data as a whole.

- Each process in application layer has to communicate with another process continuously.

- Transport layer breaks these processes down into small pieces.

Client-Server Paradigm

- A local host (client) uses a server to communicate resources.

Requirements for communication:-

- Local host

- Local process

- Remote host

- Remote process

Addressing

- We require MAC address for sending data at data link layer.

- But it is required only when the communication is not point-to-point.

- No point-to-point means that data has to traverse across multiple nodes while communication.

- Other than MAC address, the source address is also required.

- Source address: Contains information that helps in traversing of data to next node.

- In addressing, IP address is used for identifying the destination host uniquely among other hosts connected to the same network.

- While MAC address also identifies all hosts uniquely, IP address also ensures that they are connected to common network.

- Port number: Helps identifying all applications uniquely on same network.

Socket Addresses

- Socket address: Combination of IP address & port number.

- And socket address identifies individual client process uniquely.

- Address socket address: Uniquely defines a server process.

User Datagram Protocol (UDP)

- It is a transport layer protocol.

- Application’s process passes the data to network layer after compression.

- This compression is done using multiplexer on sender’s side.

- Then extraction is done using demultiplexer on receiver’s side.

- In UDP, neither receiver sends acknowledgement nor sender waits for it.

UDP Segment Header

- Connection is not established first.

- Small header size.

- No congestion control, UDPs blast away as fast as possible.

- Source port: 16-bit long port number of source.

- Destination port: 16-bit long port number of destination.

Checksum

- 16-bit long address.

- Used for detecting errors in transmitted segment.

- UDP contains header segment & checksum value.

- Checksum value: Sum of 1’s complement of all contents of segment.

- Sender calculates checksum value & assigns it to checksum value field in UDP.

- Receiver recalculates checksum value & cross checks it with the sender’s checksum.

- This cross checking is done through XORing with where error free value must be 0.

- Cross checking them tells if there is any error or not.

Applications of UDP

- Domain name services (DNS): Converts human readable domain names into IP addresses.

- Simple network management protocol (SNMP): Used for monitoring and managing devices connected to network.

- Trivial file transfer protocol (TFTP): Protocol used for transferring files between client and server.

- Routing information protocol (RIP): Used for dynamically communicating about routing information.

- Kerberos: A protocol used for aiding client-server applications for authentication of users & also provides various other security functionalities.

Transmission Control Protocol (TCP) & Internet Protocol (IP)

- Collection of protocols that connect computers to internet.

- It defines most of internet structures & functions; including packaging, addressing, sending & collecting etc.

Characteristics of TCP/IP:-

- Share data transfer: The ability to build large networks & make reliable transfer of data.

- Reliable, as it retrieves lost & misplaced data.

- Uses multiplexing.

Applications:-

- Simple mail transfer protocol (SMTP): Used for transferring email from one email address to another.

- File transfer protocol (FTP): Used for sending large sized files.

- Dynamic host configure protocols (DHCP): Assigns IP address.

- Telnet: Enables text communication via a terminal application.

- Hyper text transfer protocol (HTTP): Used for transferring web pages.

- Domain name service (DNS)

- Simple network time protocol (SNTP): Enables network devices to access time of day.

Stream Control Transfer Protocol (SCTP)

- A safe transport protocol; running parallel to unsafe one, like IP.

- Using checksums & other error checking methods, it ensures error free & non-duplicate message transfers.

- It has inherited many features of TCP including congestion management & lost packet detection etc.

Congestion Control

- Congestion happens when a huge amount of data is fed to a device, the device not being able to process it.

- Windows mechanism: Congestion management method used by TCP.

- In windows mechanism, a time limit is set on the sender to send data.

- This avoids a device from being fed excessive data.

- Steps TCP takes for congestion control:

  - Slow start

  - Congestion avoidance

  - Congestion detection

Slow start:-

- A congestion window size is given at first, measured in unit called MSS.

- At starting it is set to MSS.

- Then when the sender receives an acknowledgement, it expands the window exponentially by increasing 2’s power (2 MSS here).

- Then 4 MSS, then 8 MSS and so on.

Congestion Avoidance

- After the phase of slow start ends, phase of congestion avoidance starts.

- Here, the congestion window rises linearly.

- For each acknowledgement, congestion window size is raised by 1.

Congestion Detection

Case 1: Detection on time out:-

- Time outs before receiving acknowledgement.

- Chances are that the congestion is serious.

- Rate of congestion threshold is exponentially decreased until it becomes 1 MSS.

Case 2: Detection on duplicate acknowledgements:-

- Probability of message not being reached is less.

- Still there might be chances for it.

- Reduce the duration of congestion by half & window congestion too.

Quality of Service (QoS)

- It’s a set of traffic control mechanisms that act on the basis of network performance or the network requirements of the operator.

- It takes calculated steps to enhance network performance.

- For Qos, low performance generally means high packet delay & high number of packet loss.

Applications:-

- Video & audio conferencing/streaming.

- Hard real-time database management.

What QoS improves:-

- Delay

- Delay variation (jitter/flickering)

- Throughput

- Error rate

Types of QoS solutions:-

- Stateless solutions

- Stateful solutions

Stateless Solutions

- Doesn’t provide a very efficient control over traffic.

- But is highly scalable and resilient to possible internal damages.

- However, its unreliable of results.

Stateful Solutions

- Provides high control over traffic.

- Not much flexible or resilient for internal damages.

- Highly reliable of results.

- It also maintains the smoothness in flow of information.

QoS Improvement Techniques

- Any method that can improve service efficiency.

- Traffic shaping is one such method.

- Traffic shaping: Regulating congestion volume & rate of flow of information.

Proper traffic forming method:-

- Leaky bucket

- Token bucket

Leaky Bucket

- This method is named this because it behaves similar to how buckets behave during leakage.

- A bucket with a hole spills water at the same rate, whether more water is added to it or not & at whatever rate.

- Similarly, in this method the flow of data remains smooth & undisturbed by the number and rate of input being fed.

- High number of inputs are processed & then pushed in a stable flow.

- It basically converts a bursty flow into controlled flow.

Token Bucket

- Stricter than leaky bucket.

- If a host stays idle for too long, then resources in its bucket (queue) are emptied.

- Thus, it becomes null.

- But this method enables a host to store tokens for future use.

- Tokens: N credits provided per system clock’s tick.

- These tokens are used by the host when their chance comes for data transmission.

- One must have enough tokens for being able to send data.

- If not, then it has to wait until it has enough tokens.

- This token system regulates the traffic by fixing limited data flow across networks.