COMBINATIONAL LOGIC CIRCUITS

Half Adder

- Used for adding two 1-bit numbers.

- Provides two outputs, sum & carry.

- Sum = LSB, Carry = MSB

- Uses XOR and AND.

- Full adder and multiple adder are made using half adder only.

Mechanism of Half Adder

- XOR operation between the terms to be added decides the sum (LSB).

- And AND operation between both decides the carry (MSB).

- Input variables are called augend and addend.

- In this case, two K-Maps are required.

Half Adder Merits & Demerits

- Advantages:-

  - Simplicity (easy and not complex)

  - Speed (circuit flow)

- Disadvantages:-

  - Limited usefulness (only for two 1-bit numbers)

  - Lack of convey info (underivable)

  - Propagation deferral (slow in showing output, when input changed)

Application of Half Adder

- Arithmetic circuits (double addition)

- Data handling

- Address unravelling: Produce memory location.

- Encoder and decoder circuits.

- Multiplexers & demultiplexers.

- Counter: A clock circuit that “counts”.

Half Subtractor

- Subtraction of two 1-bit number.

- 2 inputs

- 2 output – borrow, difference

- XOR and NOT are used.

- Borrow value is A’.B

- A = minuend bit, B = subtrahend bit

- A XOR B = A’B + B’A

Half Subtractor Merits & Demerits

- Advantages:-

  - Simplicity (easy design)

  - Building blocks

  - Low cost (thus low power consumption)

  - Easy integration (derivable)

- Disadvantages:-

  - Limited functionality

  - Inefficient for multi-bits

  - High propagation delay

Applications of Half Subtractor

- Calculators (like number cruncher)

- Security alarms (detects when encounters unique circuit flow)

- Automotive vehicles

- Computer systems

Half Adder & Subtractor Using NAND & NOR

- Advantages:-

  - Universality (can perform any logic)

  - Cost-effective (cheaper than other gates)

  - Low power consumption

- Disadvantages:-

  - Propagation delay

  - Noise susceptibility (might affect circuit)

Diagrams

- Half adder using NAND

- Half adder using NOR

- Half subtractor using NAND

- Half subtractor using NOR

Full Adder

- Used for 1-bit addition.

- 3 inputs – A, B, C-IN. 2 outputs – C-OUT, S

- C means carry and S is sum.

- C-OUT:

  - Majority 1’s detector

  - Goes high when multiple inputs are high.

- Can take upto 8 inputs.

- Thus byte-wider (8 bits = 1 byte)

Cascade: One after another electrical connection.

- Full adder cascades the bit forward during calculation.

Expressions

- SUM

C-IN XOR (A XOR B)

Full Adder Using Half Adder

Full Adder Using NAND

Full Adder Using NOR

Merits & Demerits of Full Adder

- Advantages:-

  - Flexibility (can add three 1-bits)

  - Carry info (chaining)

  - Speed

- Disadvantages:-

  - Complexity

  - Propagation deferral

Applications of Full Adder

- Arithmetic circuits

- Data handling (signal/encryption/correction)

- Counters

- Multiplexers & demultiplexers

- Memory addressing circuits

- ALUs

Full Subtractor

- Names are same – minuend, subtrahend

- 3 inputs – A, B, Bin and 2 outputs – D, Bout

- D is difference whereas B means borrow.

Calculation

- Step 1: Convert the numbers to 2’s complement form, in case of negative number.

- Step 2 to n: Do simple subtraction of the numbers step by step.

Logic Diagram of Full Subtractor

- Consists of 2 half subtractor and an additional OR gate.

- The OR gate calculates the Bout bit.

D = (A XOR B) XOR Bin

Bout = Bin (A XOR B)’ + A’B

K-Map for Full Subtractor

Code Converters

- Excess-3: Unweighted, self-complementary code.

- Self-complimentary: 1’s complementary of code is excess-3 for 9’s complement of other code.

- For excess-3 code for a given number’s complement, we just need to find 1’s complement of the given number’s excess-3 code.

- It is also excess-3 code for the 9’s complement of 3.

Decimal to Excess-3 Conversion

- Step 1: Add 3 to each digit of the decimal number.

- Step 2: Now convert the number into 4-bit binary.

- We mark numbers after 9 as X (as decimal system is 0 to 9).

BCD(8421) to Excess-3

- Just add binary 3 (11) to the BCD number.

- Drawing K-Map Using Truth Table:

- Corresponding digital circuit:

Excess-3 to Decimal Conversion

- Exactly reverse of decimal to excess-3.

Excess-3 to BCD(8421)

- Exactly reverse of BCD(8421) to Excess-3.

- Negative numbers are marked as XXXX.

- K-Maps are created for separate letters:

- After creating any equation, minimizing it is a very good practice.

- Logic circuit for the same:

- E3 to E1 = w to z

- B3 to B1  = A to D

Gray Code

- Successive pair of numbers differ by 1-bit.

- Thus, used for correcting errors.

- Alternative names for Gray code:-

  - Unity Hamming Distance Code

  - Cyclic Code

  - Reflecting Code

Binary to Gray Conversion

- Logic circuit:-

Binary to Gray

- Logic circuit:-

BCD to 7 Segment Decoder

- BCD: Encoding decimal numbers as 4-bit binary numbers.

- 7 segment display: 7 LEDs connected in a manner to represent all hexadecimal numbers (decimal only in our case).

7 Segment Display Types

- Common cathode type:

  - Cathodes of all 7 LEDs are connected together to ground (or -Vcc).

  - LED displays digits when HIGH signal is supplied to anodes.

- Common anode type:

  - Anodes of all 7 LEDs are connected together to battery (or +Vcc).

  - LED displays digits when LOW signal is supplied to cathodes.

Process of Working of 7 Segments

- First, the decimal number is converted to BCD signal.

- Then the BCD codes communicate with the 7 segment LEDs.

Structure of 7 Segments

BCD to 7 Segment Decoder

- For common anode type:-

- For common anode type:-

  - Just reverse the result outputs in cathode type.

  - Whatever we do, making K-Map and minimizing the equations is a good practice.

K-Map for 7 Segment

- We make 10 combinations in its truth table.

- For rest combinations shown in K-Map by default must be marked as don’t care conditions.

- For segment #a:-

Parallel Adder

- Used to calculate addition of numbers more than 1-bit in length.

- Consists of full adders connected in chain.

Number of bits = Number of full adders

Parallel Subtractor

Merits & Demerits of Parallel Adder/Subtractor

- Advantages:-

  - Faster than serial adder/subtractor.

  - Time for calculation doesn’t depend on number of bits.

  - Less costly.

- Disadvantages:-

  - Each adder has to wait for previous carry.

  - Propagation delay.

Carry Look-Ahead Adder

- There are many ways to reduce propagation deferral.

- One of the way is carry look-ahead.

- Carry look-ahead: Calculates carry signals in advance.

- Full carry adder is also known as ripple carry adder (just write Cn instead Cout).

- A block waiting for previous block to pass signals is one type of propagation delay.

Propagation time = Propagation delay for other blocks

- It reduced propagation deferral by sending signals to group of two bits at a time, rather than one.

Carry Generator Figures

Time Complexity Analysis (Carry Look-Ahead Adder)

- It is made up of two parts:

  - Part that calculates carry for each bit.

  - Part that adds two bits and carry.

- Time complexity: log(n)

  - It generates from the bit generating part.

  - For k inputs, it is logk(n+1).

Merits & Demerits Of Carry Look-Ahead Adder

- Advantages:-

  - Propagation delay reduced.

  - Fastest addition logic

- Disadvantages:-

  - Gets complicated as the number of variables increases.

  - Costlier circuit arrangement.

Note About Carry Generator

- For addition of n-bit variables it requires:

  - [n(n+1)]/2 AND gates.

  - n OR gates.

Magnitude Comparator

- Compares two binary numbers.

- 3 output terminals: A>B, A=B, A<B

- The circuit checks starting from MSB to LSB, and terminates when A>B or A<B is encountered.

Ways to Implement Magnitude Comparator

- By using XOR, AND, OR gates.

- Arrangement of full adders.

- Formula obtained:

(A<B)+(A>B) = A’B+AB’

=> ((A<B)+(A>B))’ = (A=B)

Magnitude Comparator as per Bits

- 1-bit each: Inputs are A and B.

- 2-bit each: Inputs are A0, A1, B0 and B1.

- Number of output terminals are 3 always.

K-Map for A>B (2-bits)

- For any given truth table for any topic, there might be multiple outputs columns.

- For each output column, always separate K-map is created.

Circuit for 8-bit Comparator

Number of total combinations = 22n

Number of equal combinations = 2n

Number of unequal combinations = (Subtraction of above two)

Number of A<B or A>B = Enequals/2

Cascading Comparator

- Used for comparing digital numbers with length greater than 4-bits.

- The body on left is called lower-order comparator.

- And the one right is called higher-order comparator.

Applications of Comparators

- Are used in CPUs and MCUs (microcontrollers).

- Control and detection applications.

- Servo motor controls.

- Password verification & biometrics system.

BCD Adder

- Adds BCD numbers.

C' + S3'.S2' + S3'.S1' = 1

Encoders And Decoders

- Encoder:-

  - Used for converting one form of information to another.

  - Converts 2n lines of input into a code of n output.

- Decoder:-

  - Same use in reverse.

  - Converts n lines of input into a code of 2n outputs.

- Encoder example:-

Boolean Function for Outputs

X = D4 + D5 + D6 + D7

Y = D2 +D3 + D6 + D7

Z = D1 + D3 + D5 + D7

Limitations to Encoders

- Limitation is that only one output can be active for a single high input.

- Else for multiple high inputs, the output is undefined.

- To avoid similar outputs when multiple inputs are high, we use priority encoders.

- Also, all inputs can’t be 0, where we get output same as D0.

- For avoiding it, an extra digit of bit is added, called valid “V” bit i.e., 0.

- Don’t care conditions are used for main outputs in such cases, and V-bit is 0 there.

- Otherwise in other scenarios where output is decided, it is 1.

- V-bit is 0 only when all inputs are 0.

4-to-2 Encoder

Decoders

- For example:- 3-to-8 decoder.

- Truth table drawn totally opposite.

Encoders

- Active: low

- Inactive: high

- In encoder, only one output is active low.

- Converts binary inputs into a certain binary code, of n bit.

- Types of encoders:-

  - 4 to 2 encoders

  - Octal to binary encoders (8 to 3 encoder)

  - Decimal to BCD encoder

  - Priority encoder

Some Encoder Diagrams

- 4 to 2 encoder:

- 8 to 3 encoder:

- Decimal to BCD:

  - 10 inputs, 4 outputs

  - Truth table similar to other encoders.

- Priority encoder:

  - Similar to 4 to 2 encoder.

  - But the triangle below the 1s are don’t care type.

Application of Encoders

- Most digital circuits.

- Basic arithmetic operations.

- Detecting interrupts in microprocessors.

Merits & Demerits of Encoders

- Advantages:-

  - Reduction in number of lines (more inputs to less outputs)

  - More reliability (less error, due to 1st point)

  - Improved performance (reduced transmission time)

- Disadvantages:-

  - More complex circuit.

  - Limited application.

  - Less flexible (fixed number inputs and outputs)

Binary Decoder

- Converts serial codes to parallel outputs.

- Only one input is low active.

- Input of n-bit and 2n or less output lines.

- Only one output is activated, others deactivated.

- Unused combinations = Less than 2n outputs

- Each combination of input provides a unique output.

2-4 Decoder

- Only one output line is HIGH, determines (decodes) the output code.

- An extra input called Enable, denoted by E, controls ON and OFF function of the system.

- Enable must always be HIGH in order for the system to work.

Merits & Demerits of Binary Decoders

- Advantages:-

  - Increased flexibility

  - Improved performance

  - More reliability (reduced error chances)

- Disadvantages:-

  - Increased complexity

  - Limited applications

  - Limited outputs

Application of Binary Decoder

- Memory assigning in computers

- Control circuits

- Display drivers

- Error rectification

Combinational Circuits Using Decoder

- A decoder with Enable input acts as demultiplexer.

- So compulsorily 2n output lines (2n minterms).

- Demux using decoder:

- Full adder can be constructed by using 3:8 decoder and OR gates.

Merits & Demerits of These

- Advantages:-

  - Simplification

  - Flexibility

  - Modularity

  - Reliability

- Disadvantages:-

  - Complexity

  - Delay

  - Power utilization

  - Limited adaptability

Multiplexers

- Also known as:-

  - n selector

  - Parallel to serial converter

  - Many to one circuit

  - Universal logic circuit

- Used for increasing the amount of data that can be carried within certain amount of time and bandwidth.

- Selector pins are also known as control lines.

Gates Using Muxes

- AND gate using mux:

- Implementation of NAND, NOR, XOR, XNOR gate requires two 2:1 mux.

- NAND:

- NOR:

- XOR:

Using Mux for Creating Larger

For example: Creating 64:1 mux using 4:1.

64:1 -> m:1, 4:1 -> n:1

Continuous m/n until answer is 1.

64/4 = 16

16/4 = 4

4/4 = 1

We did so 3 times, so 3+1 = 4. Four 4:1 mux are required for making a 64:1 mux.

Merits & Demerits of Multiplexers

- Advantages:-

  - Space-saving

  - Cost-successful

  - Time-saving

  - Flexibility

- Disadvantages:-

  - Limited input numbers

  - Delay

  - Complex connections

  - Power utilization

Hazards

- Temporary fluctuation in digital device.

- Gets resolved by itself.

- Updates wrong output momentarily due to propagation delay.

- Of Three Types:-

  - Static hazard

  - Dynamic hazard

  - Function hazard

Static-1 Hazard

- Occurs in SOP circuits.

- Output is 1, but momentarily changes to 0 before becoming 1.

Static-0 Hazard

*you know it now*

Static Hazards on K-Map

- Mark pair of cells that do not occur to be in same group (prime implicants).

- Each such pair is static-1 hazard.

- Some changes in circuit are made in order to fix hazard.

- Similarly, in static-0 hazard we consider 0s instead 1s.